Eco Audit Report.. Product name.; Cat Teaser Wand (1).. Country of manufacture.; The Whole World... Country of use.; The Whole World.. Product life (years).; 1.. Summary:.... Energy details.; Climate change (CO2-eq) details.; Cost details.. Phase; Energy (MJ); Energy (%); Climate change (CO2-eq) (kg); Climate change (CO2-eq) (%); Cost (GBP); Cost (%). Material; 2.51e+05; 78.8; 9.87e+03; 67.4; 4.73e+03; 42. Manufacture; 5.83e+04; 18.3; 4.08e+03; 27.9; 6.08e+03; 54. Transport; 7.85e+03; 2.5; 597; 4.1; 431; 3.83. Use; 0; 0.0; 0; 0.0; 0; 0. Disposal; 1.45e+03; 0.5; 102; 0.7; 15.8; 0.14. Total (for first life); 3.19e+05; 100; 1.46e+04; 100; 1.13e+04; 100. End of life potential; -3.16e+04; -992..

|  | Eco Audit Report.. Summary.. Energy Analysis.... Energy (MJ/year). Equivalent annual environmental burden (averaged over 1 year product life):; 3.19e+05... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Energy (MJ); %. Wand (Handle); ABS (medium-impact, injection molding); Virgin (0%); 0.05; 50000; 2.5e+03; 2.4e+05; 96.8. String; Hemp fiber; Virgin (0%); 0.015; 50000; 7.5e+02; 8.1e+03; 3.2. Total; 100000; 3.3e+03; 2.5e+05; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; Energy (MJ); %. Wand (Handle); Polymer molding (injection, blow); -; 2.5e+03; kg; 5.6e+04; 96.7. String; Fabric production; -; 7.5e+02; kg; 2e+03; 3.3. String; Cutting and trimming; -; 0; kg; 0; 0.0. Total; 5.8e+04; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); Energy (MJ); %. China to UK; Sea, container ship; 1.8e+04; 7.9e+03; 100.0. Total; 1.8e+04; 7.9e+03; 100.. Breakdown by components.. Component; Mass (kg); Energy (MJ); %. Wand (Handle); 2.5e+03; 6e+03; 76.9. String; 7.5e+02; 1.8e+03; 23.1. Total; 3.3e+03; 7.9e+03; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Energy (MJ); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Energy (MJ); %. Wand (Handle); Downcycle; 80.0; 1.1e+03; 75.7. String; Downcycle; 90.0; 3.5e+02; 24.3. Total; 1.5e+03; 100.. EoL potential:.. Component; End of life option; % recovered; Energy (MJ); %. Wand (Handle); Downcycle; 80.0; -3.1e+04; 99.8. String; Downcycle; 90.0; -68; 0.2. Total; -3.2e+04; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. Climate Change (CO2-eq) Analysis.... CO2 (kg/year). Equivalent annual environmental burden (averaged over 1 year product life):; 1.46e+04... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Climate change (CO2-eq) (kg); %. Wand (Handle); ABS (medium-impact, injection molding); Virgin (0%); 0.05; 50000; 2.5e+03; 9.2e+03; 93.4. String; Hemp fiber; Virgin (0%); 0.015; 50000; 7.5e+02; 6.5e+02; 6.6. Total; 100000; 3.3e+03; 9.9e+03; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; Climate change (CO2-eq) (kg); %. Wand (Handle); Polymer molding (injection, blow); -; 2.5e+03; kg; 3.9e+03; 96.2. String; Fabric production; -; 7.5e+02; kg; 1.6e+02; 3.8. String; Cutting and trimming; -; 0; kg; 0; 0.0. Total; 4.1e+03; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); Climate change (CO2-eq) (kg); %. China to UK; Sea, container ship; 1.8e+04; 6e+02; 100.0. Total; 1.8e+04; 6e+02; 100.. Breakdown by components.. Component; Mass (kg); Climate change (CO2-eq) (kg); %. Wand (Handle); 2.5e+03; 4.6e+02; 76.9. String; 7.5e+02; 1.4e+02; 23.1. Total; 3.3e+03; 6e+02; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Climate change (CO2-eq) (kg); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Climate change (CO2-eq) (kg); %. Wand (Handle); Downcycle; 80.0; 77; 75.7. String; Downcycle; 90.0; 25; 24.3. Total; 1e+02; 100.. EoL potential:.. Component; End of life option; % recovered; Climate change (CO2-eq) (kg); %. Wand (Handle); Downcycle; 80.0; -9.9e+02; 99.5. String; Downcycle; 90.0; -4.7; 0.5. Total; -9.9e+02; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. Cost Analysis.... Cost (GBP/year). Equivalent annual environmental burden (averaged over 1 year product life):; 1.13e+04... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Cost (GBP); %. Wand (Handle); ABS (medium-impact, injection molding); Virgin (0%); 0.05; 50000; 2.5e+03; 4e+03; 85.5. String; Hemp fiber; Virgin (0%); 0.015; 50000; 7.5e+02; 6.8e+02; 14.5. Total; 100000; 3.3e+03; 4.7e+03; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Country of manufacture.; The Whole World.. Component; Process; Length (m); % Removed; Amount processed; Cost (GBP); %. Wand (Handle); Polymer molding (injection, blow); -; -; 2.5e+03; kg; 6.1e+03; 100.0. String; Fabric production; -; -; 7.5e+02; kg; 0; 0.0. String; Cutting and trimming; -; -; 0; kg; 0; 0.0. Total; 6.1e+03; 100... Transport:.; Summary.. Package dimensions.. Height (m); Width (m); Depth (m). 1; 0.03; 0.02... Breakdown by transport stage.. Stage name; Transport type; Distance (km); Cost (GBP); %. China to UK; Sea, container ship; 1.8e+04; 4.3e+02; 100.0. Total; 1.8e+04; 4.3e+02; 100.. Breakdown by components.. Component; Mass (kg); Cost (GBP); %. Wand (Handle); 2.5e+03; 3.3e+02; 76.9. String; 7.5e+02; 1e+02; 23.1. Total; 3.3e+03; 4.3e+02; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Cost (GBP); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Cost (GBP); %. Wand (Handle); Downcycle; 80.0; 12; 76.9. String; Downcycle; 90.0; 3.6; 23.1. Total; 16; 100... Notes:.; Summary.... |
| --- | --- |